# ARISS Project Selection and Use Committee

# Slide Show Module Submission

August 4, 2015

## Art Towslee:

  - ARISS DATV Team

  - Support the ARISS DATV Project

  - Art Towslee W8DMR , towslee1@ee.net

## PROJECT DESCRIPTION:

The module described here is a small “Slide Show” metal enclosure (approx. 5.5” high x 3.5” wide x 2” deep) that plugs directly into the existing onboard HamVideo module connectors by means of the two existing onboard power and data cables. The main purpose is to provide a source of composite video in a sequence of fixed pictures for the HamVideo transmitter. That way actual video is transmitted when an active school contact is not in progress. In addition, the internal camera and LCD display allow live video or used to create additional fixed pictures for the “slide show” sequence. A three position switch is provided in order to select “slide sequence”, “live video” or “take a picture” positions. If audio is needed, it has a microphone input in order to send live audio. The slide pictures can be entered by means of a thumb drive plugged into the USB port or uploaded from the ground via the Ethernet port.

  - Project design:

Cost and construction of first prototype was made by Art Towslee, a member of the ARISS HamVideo development team. Art will construct and pre-test a second prototype after an initial design review of the first one. The second prototype will be made to flight module standards. Further costs for a flight model will be made by a combined effort of NASA, ESA and ARISS. Art will contribute construction and testing efforts as needed within his capabilities.

  - Operation modes:

2.4.1 Power ON-OFF front panel guarded sw. disconnects 28VDC power source.

2.4.2 Mode switch. Three position switch for Slide-Camera-Shutter positions.

2.4.2 Crew requirements. Other than power on, picture upload can be accomplished by ground personnel. Crew involvement is needed only if camera snapshots are required.

  - Project status:

2.5.1 Hardware:

One functional first prototype was designed and constructed by Art Towslee. During this construction some desired changes were noted. It is the intention that this existing first prototype be evaluated by relevant personnel to identify features desired and any non-compliance. After the review process Art will construct a second prototype to flight standards for both hardware and software testing. Formal Autocad documentation will be done at that time. (Art has access to a certifiable environmental testing lab with anechoic chamber to conduct pre-compliance RFI emission tests. Results will determine if hardware changes are needed before any NASA or ESA testing is started.

2.5.2 Software:

Software is contained in a removable SD card in the Raspberry PI computer module within the Slide Show module enclosure. Uploaded pictures from the Ethernet will be stored in this module. Presently there are some bugs which are in the process of being identified and resolved. The Raspberry Pi development team is assisting in this area. At some point, the software needs NASA/ESA review because the unit will be connected to the onboard Ethernet for the picture upload process.

## ISS AND GROUND INVOLVEMENT:

  - The crew will not be involved in project development. Once the module is in the ISS, crew initially will be required to connect the two power and data cables (already onboard) between the HamVideo and Slide Show modules. Then the crew must connect the Slide Show module Ethernet port to the onboard Ethernet cable. Once set up, the crew needs only to turn the ON/OFF switch to “ON” position and MODE switch to “Slide” position to start slide show.

  - Power for the Slide Show module is supplied from the 28VDC power connector on the HamVideo module.

  - Ground personnel will be required to upload new picture files to the Slide show module via the onboard Ethernet port. Crew will not be involved.

  - Slide Show module can be stowed with the HamVideo module or left connected to existing HamVideo data and power cables. Velcro will be on back of module.

  - EVA’s will not be required in any way to install or operate the module.

## EDUCATION:

  - When the Slide Show module is operational, it will provide an actual video data transmission to help test ground tracking equipment. Also, it will be instrumental in creating new tracking stations as well as a teaching tool for schools.

## AMATEUR RADIO BENEFITS:

  - The Amateur Radio community will benefit because it will provide meaningful data to receive thereby creating more interest, recognition and create more tracking stations.

## PROBLEM SOLUTIONS:

  - Presently the HamVideo transmitter is sending blank “no video” signals due to a software error in existing transmit module. The Slide Show video will create an actual video signal during the times between actual educational sessions.

## FREQUENCY COORDINATION:

  - No new frequencies are involved.

## EQUIPMENT REQUIREMENTS:

  - Slide Show module is designed for continuous unattended operation.

  - Module powers up to an operational state. The boot process is automatic.

  - Case temperature rise is less than 10 degrees C

## APPENDIX:

  - Block diagram