# ARISS Project Selection and Use Committee

Proposal Submission:

# Interoperable Radio System Upgrade

# (D710 and Low Voltage Power Supply for Columbus Module)

August 4, 2015

## Organization making proposal:

  - ARISS

  - Organization purpose:  Promote Amateur radio and educational activities on the International Space Station

  - Point of contact and contact information (email, phone)

Lou McFadin, W5DID	arissw5did@comcast.net

## Project Description:

  - This power supply, Kenwood D710 transceiver, and associated cables are proposed as a permanent addition to the Amateur Radio equipment onboard the International Space Station. It will be located in the Columbus Module and replace the Ericsson hand-held radio and its components currently on the ISS.

  - Rough cost estimates indicate that this project will cost about US$82,000 not including the cost of developing the safety data package.

  - The attached diagram shows the general way the system will be configured.

  - The System provides the following features.

    - System is flexible and forward-looking, providing capabilities that should be applicable to future systems in the Columbus Module.

    - System provides interoperability between Russian segment (Service Module/FGB) and US Operating Segment (Columbus Module).

      - All systems can be powered in either location, with proper cabling.

    - Provides two 13.8 VDC power outlets, one dedicated to operate the D710 radio.

    - Provides four USB standard 5 VDC charging stations, 2.1 A each.

    - Provides for a 120VDC pass-through connector so that the power supply can be operated without solely occupying the ISS 120VDC outlet.

    - Provides three 28VDC outlets with the standard outlet connector that was used on the Shuttle program. This connector is the same one that is now used on the ISS-Ham packet module.

    - Provides one 28VDC outlet with a connector that is used as a standard 28VDC outlet on the Russian modules. This is compatible with existing cables that are already on the ISS.

    - Can be operated from either the 120VDC outlet on the ISS or any 28VDC power source such as the PPS or the Russian Service module.

    - Provides a 28VDC pass-through capability

    - The Kenwood D710 provides a significant improvement in the Ham radio capabilities in the Columbus module on the ISS.

      - Higher power (50 W versus 6W now with the Ericsson radio).

      - More programmability (1000 memory channels).

      - Five program modes so that pre-programed settings can be set up for the crew, thereby simplifying their work.

    - System is compatible with Ham TV in the Columbus Module.

    - Cabling will be provided to allow use of existing David Clark headset with D710

  - Certification process - TBD

## ISS and Ground involvement:

  - The crew will need to be trained on the operation of the D710 radio and its associated equipment.

  - This system can be operated from either 120VDC on the Columbus module or 28VDC on either the Columbus module or any other 28VDC source such as the PPS or the 28VDC in the Russian segment.

  - There will be no new facilities or equipment required on the ground. Occasionally there may be a need to reconfigure the radio. In that case, a file will need to be uploaded to the ISS for reloading the radio parameters and frequencies. The equipment to accomplish this is still to be determined.

  - This equipment will be located in the Columbus Module and will also be designed and certified for use in the Russian Service module.

  - No EVA is required to install this equipment.

## Education:

  - The educational benefits if the ARISS program are well documented in other documents.

## Amateur Radio Benefits:

  - This new equipment will make it easier for the crew to contact hams on the ground and thereby create great interest in ham radio on the ISS for many of the world’s Amateur radio operators.

## Problem Solutions:

  - The higher power levels will solve the problem of weak signals being heard from the ISS.

## Frequency Coordination:

  - This equipment will operate on the 144 to 145MHz and 430 to 440 MHz amateur space to ground and ground to space allocated frequencies.

## Equipment Requirements:

  - Equipment will be capable of operating unattended for extended periods (weeks).

  - Equipment will be designed to power on into the default state.  A reset of the equipment will restore it to a desired default state.

  - Crew interfaces will be very simple, closely matching the equipment already onboard.

  - Current procedures will be modified for equipment operation prior to being approved for flight.

  - System is potentially capable of being controlled from the ground, pending additional hardware and software development.

  - There is no assumption that equipment will be returned after its use on orbit is complete.

  - All drawings and documentation will be provided to ARISS-I.

## Other:

  - No additional information has been identified so far.