Interoperable Radio System Upgrade

August 4, 2015

Prepared by Mark Steiner

Overview:  In January 2015 a team investigated the replacement of the Ericsson radio currently in the Columbus Module with a more capable radio. The top candidate was to provide a Kenwood D710 radio with any needed power supply modules. The team has been meeting roughly twice a month through 2015, and the result is a proposal for consideration by the Project Selection and Use Committee with a recommendation to the ARISS-I delegates for consideration at the face to face meeting in Japan in August 2015.

Members:  The team consists of the following members:

Mark Steiner		Chair

Lou McFadin		ARISS Chief Engineer

Stefan Wagener	Member

Dave Taylor		Member

Kenneth Ransom	Member

Kerry Banke		Member

Frank Bauer		Ex-officio

Actions:  The team developed a top-level design considerations document that was the basis for an input to a proposal to CNES to obtain a dedicated Portable Power Supply (formerly known as “KUPS”) that would convert the ISS 120 VDC to 28 VDC. Since that proposal, the team has refined the design to consider two options for power supplies, the use of a dedicated PPS and the use of an ARISS-developed power supply.

The result is a robust system that will provide interoperability in the Russian Segment (FGB and Service Module) and the US Operating Segment (Columbus Module). Any portion of the proposed system could be operated in either segment with proper cabling. It would also allow any other ARISS systems that are certified in one segment to be operated in the other segment, after proper certification.

Top-level system requirements:

- Launch Kenwood D710 radio and supporting elements so ARISS-I has a high-power radio in the Columbus Module (COL) that is compatible with the D710 radio in the Russian Service Module (SM).

- All components shall be able to be used interoperably across all the ISS segments.

- Include a power supply that can interface with the existing system in the COL (including the Ham TV) and be flexible enough for future systems.

- Be compatible with the other ISS Ham system components (headsets, SSTV module, antennas, etc.), with connector adapters, if necessary.

- Allow D710 to be re-programmed on-orbit. Options to be considered are an Apple iPad with standard ISS software load or a Windows tablet.

- Provide enough flight-like hardware for training and system certification.

- Include power system pass-throughs to maintain flexibility to bring in other options/future systems.

- Provide flexibility for current and future capabilities/modes:

  - Cross-band repeater

  - SSTV

  - Packet/APRS/Other digital techniques

- Minimize crew time

  - Provide options for possible later addition of remote control of system from the ground (provide interfaces, not necessarily implement at this time)

- Minimize volume in ISS COL

- Ensure systems meet ISS and equipment thermal control

  - Power supply

  - D710

System Description:

The proposal consists of a separate, ARISS-developed and built Low Voltage Power Converter that includes the capability to convert directly from the ISS 120 VDC; a Kenwood D710 transceiver; necessary cables; certifications; and procedures. See Fig. 1.

Fig. 1 - Proposed system block diagram

ARISS Low Voltage Power Converter description:

This unit converts either the 28 VDC provided by the PPS or the 120 VDC ISS provides to the voltages needed for the actual inputs of the rest of the ARISS-I hardware. It would provide 13.8 VDC for the Kenwood D710 and 5 VDC at 2.1 A for a set of four standard USB connectors for charging USB devices. It would also provide pass-throughs for the 120 VDC and 28 VDC for additional equipment connections (so we do not tie up a 28 VDC connector), 3 standard US 28 VDC connectors and one standard Russian 28 VDC connector. This would allow for the interoperability of Russian hardware in the Columbus Module.

Fig. 2 – Low Voltage Power Supply

D710 considerations: The Kenwood D710 in the COL will have some differences from the D710 that is currently in the Russian Segment:

- It will have a heat sink and fan to allow continuous operation. That may include a firmware change to allow overriding the 3 minute timeout.

- It will have a simpler command set for entering the cross band repeat mode.

- It will have different connectors for power, etc.

- We may include a firmware modification to reduce maximum power output to 25W.

Launch mass estimates for whole system:

|  | Mass (kg) |
| --- | --- |
| PPS | 3.97 |
| 28V Low-Voltage Power Supply | 4.00 |
| Kenwood D710 | 1.86 |
| Cables | 2.00 |
| Total | 11.83 |

Other considerations:

For each flight component (except PPS) there shall be the following 7 units, all identical to the flight unit:

1 - Flight

1 - Flight spare

3 - Training (1 for US/JSC and 2 for Russia)

2 - Engineering (2 units), for trouble-shooting, procedure development, etc.